FeCIM Component Spec: Control FSM and Config Register File

Module: control_fsm, config_regs, result_buffer, display_driver Owner: Lane A (RTL), register map frozen jointly with Lanes B and C Depends on: packet_parser (command dispatch), mac_array (lane control) Milestone: Week 4 Status: draft — register map freezes in week 1

1. Purpose and scope

The sequencer for the whole design. Owns the config register file, drives the MVM sequence across the lane array, arbitrates weight memory access, buffers results, and runs the on-board display.

Everything with a state that outlives a single packet lives here. The parser is stateless between packets; the array is a pure datapath; this module is where the machine remembers what it is doing.

2. Correction to the earlier latency estimate

The project plan quoted 2.7 µs for a 64×64 MVM based on 128 issue cycles. Working through the actual sequence, that number was too low — it counted only the accumulate cycles and ignored pipeline drain and accumulator readout.

| Phase | Cycles | Note |
| --- | --- | --- |
| Clear accumulators | 1 | per pass |
| Issue rows | 64 | per pass, R cycles |
| Pipeline drain | 3 | lane pipeline depth |
| Drain accumulators | 32 | per pass, L cycles |
| Per pass | 100 |  |
| Two passes (64 cols / 32 lanes) | 200 |  |

200 cycles at 50 MHz = 4.0 µs. Use this number in the report, not 2.7 µs.

The conclusion is unchanged and if anything strengthened: the array is still four orders of magnitude faster than the 360 ms UART weight load, so end-to-end timing is entirely host-bound. But the report should quote a number that was derived rather than one that was estimated, and the derivation belongs in the documentation.

3. Column-to-lane mapping

The mapping choice determines the address decode, so fix it before anything else.

Lane l owns columns {l, l+L, l+2L, ...} — interleaved, not blocked. On pass p, lane l computes column c = p·L + l.

Weights live in each lane's private M9K at local address {p, r} — pass index in the upper bits, row index in the lower.

3.1 The address decode is free

Because the row count is a power of two, the sequence counter is the weight address. With R = 64 and L = 32:

seq_cnt : 0 .. (passes·R - 1)              // 0..127 for a 64×64 tile

row_idx     = seq_cnt[5:0]                 // wire slice

pass_idx    = seq_cnt[6]                   // wire slice

weight_addr = seq_cnt                      // identity — no logic at all

act_addr    = seq_cnt[5:0]                 // wire slice

One counter drives the entire compute sequence. No multipliers, no adders, no decode logic.

The same property makes the bulk write decode free. Given bulk_addr = c·R + r from the host, sending weights column-major:

r          = bulk_addr[5:0]

c          = bulk_addr[15:6]

lane_sel   = c[4:0]                        // c mod 32

p          = c[15:5]                       // c div 32

local_addr = {p, r}

All wire slicing. This is why R and L must stay powers of two. If someone proposes a 100-row tile to match a dataset, the cost is a divider in the address path and a multiplier in the sequencer. Pad to 128 instead and mask the unused rows.

4. Compute sequence

IDLE

│  start_compute

▼

CLEAR ─────────────────┐

│  1 cycle           │

▼                    │

ACC                   │  pass_idx < passes-1

│  R cycles issue    │

│  + 3 drain         │

▼                    │

DRAIN ─────────────────┘

│  L cycles

│  pass_idx == passes-1

▼

DONE ──► IDLE   (resp_done to packet_tx)

4.1 CLEAR

Zero all lane accumulators. One cycle, broadcast.

Optimization, if you want it: fold this into ACC by muxing the accumulator input to select 0 instead of the running sum on row_idx == 0. Saves one cycle per pass and one state. Costs a 24-bit 2:1 mux per lane. Worth doing only if you are chasing cycles, which you are not — implement CLEAR as its own state first and optimize later if ever.

4.2 ACC

Issue one row per cycle: drive weight_addr to all lanes, act_addr to the activation buffer, assert accumulate enable. Run for active_rows cycles.

Then hold for 3 more cycles before leaving the state. The lane pipeline is BRAM read → noise inject → multiply → accumulate, so the last issued row's contribution has not reached the accumulator when the counter finishes. Leaving ACC early silently drops the last three rows from every result — a bug that produces plausible numbers and is miserable to find. Make the drain count a named parameter tied to the lane pipeline depth, so that if Lane A adds a pipeline stage the constant follows.

localparam LANE_PIPE_DEPTH = 3;   // must track mac_lane.v

4.3 DRAIN

Move L accumulator values into the result buffer, one per cycle.

Use a shift chain, not a multiplexer. The obvious implementation is a 32-to-1 mux of 24-bit values, but that is 744 LEs with five levels of logic delay on a path that spans the whole array physically. Instead, add a 2:1 mux on each accumulator input selecting between "accumulate" and "take the neighbor's value", and shift the whole array toward lane 0 one position per cycle.

lane31 ──► lane30 ──► ... ──► lane1 ──► lane0 ──► result_buffer

Same area (~768 LEs), but every connection is lane-to-adjacent-lane, so placement is local and timing is trivial. This matters more than it looks: the multiplexer version is the most likely thing in the design to fail timing when you scale to 64 lanes.

4.4 DONE

Assert completion to the parser, which sends the COMPUTE response. Latch the cycle count into CYCLE_CNT for the host to read.

5. Weight memory arbitration — there isn't any

M9K blocks are true dual-port. Bulk writes use port A of the selected lane; compute reads use port B of every lane. The two never contend.

Mutual exclusion also holds at the protocol level: the host is in request/response lockstep, so a WRITE_WEIGHTS packet has been fully acknowledged before a COMPUTE packet is sent. The FSM still rejects start_compute outside IDLE with status 0x04, because relying on host politeness for correctness is how you get a bug that only appears when someone writes a threaded driver.

Since writes and reads never overlap, M9K read-during-write behavior is irrelevant here and needs no attention in the instantiation.

6. Config register map

Little-endian, 32 bits per register, byte-addressed by index. Freeze this in week 1 — Lane B's model and Lane C's driver both encode it.

| Addr | Name | Access | Description |
| --- | --- | --- | --- |
| 0x00 | CTRL | W | [0] soft reset, [1] clear results, [2] reseed LFSRs |
| 0x01 | TILE_CFG | RW | [15:0] active rows, [31:16] active cols |
| 0x02 | QUANT_LEVELS | RW | conductance levels, 2–256 |
| 0x03 | D2D_SIGMA | RW | device-to-device variation scale (Q8.8) |
| 0x04 | READ_SIGMA | RW | cycle-to-cycle read noise scale (Q8.8) |
| 0x05 | NOISE_SEED | RW | LFSR seed, 0 is remapped to 0xACE1 |
| 0x06 | NOISE_EN | RW | [0] quant [1] d2d [2] read [3] stuck [4] IR [5] ADC |
| 0x07 | ADC_BITS | RW | output truncation width, 4–24 |
| 0x08 | STUCK_RATE | RW | stuck-at injection rate |
| 0x09 | BAUD_INC | RW | UART fractional divider increment |
| 0x0A | STATUS | R | [3:0] FSM state, [4] busy, [5] last error |
| 0x0B | CYCLE_CNT | R | cycles taken by the last COMPUTE |

6.1 NOISE_EN is the most important register in the design

Every non-ideality is individually maskable. This is what makes the whole project scientifically legible rather than a black box: you can run the identical weights and activations with each effect isolated, and attribute accuracy loss to a specific physical mechanism.

It is also what makes verification tractable. Lane B's equivalence tests run with NOISE_EN = 0 to prove the clean datapath is bit-exact against the model, then enable one bit at a time. Without this register you would be debugging six interacting effects simultaneously.

6.2 TILE_CFG and power-of-two addressing

active_rows and active_cols limit how many rows the sequencer issues and how many columns are drained, letting you sweep tile sizes without a rebuild.

They do not change the address stride. Weight addressing always uses the physical R_MAX = 64 stride, so a 48-row configuration uses addresses 0–47 of each 64-entry region and leaves 48–63 unread. Non-power-of-two active sizes are therefore free; the physical tile stays a power of two and the wire-slice decode of §3.1 is preserved.

6.3 Reset defaults

On reset: TILE_CFG = full tile, NOISE_EN = 0, QUANT_LEVELS = 256, all sigmas = 0, BAUD_INC = 115200.

Reset means an ideal, noiseless crossbar. A freshly configured board computes exact integer MVMs, so if the first thing you see after programming is wrong, the problem is the datapath and not a noise setting someone left enabled.

7. Result buffer

C × 24 bits — 64 × 24 for the MVP. One M9K in 256×32 mode holds 256 results with room for a 128-column tile and 32-bit accumulators.

Written one word per cycle during DRAIN, read by the parser during READ_RESULT. Single-port is sufficient since the two phases never overlap.

Results persist until the next COMPUTE, so the host can re-read them without recomputing — useful when a response packet is lost and the driver retries.

8. Display

Not decorative. This is the physical interface M0 asks for in an Infrastructure project, and it is the only debug channel that keeps working when the serial link is the thing that is broken.

| Element | Shows |
| --- | --- |
| HEX5–HEX4 | FSM state (hex) |
| HEX3–HEX0 | result[0], or RX byte count, selected by SW[0] |
| LEDR[9] | busy |
| LEDR[8] | error latched since reset |
| LEDR[7:0] | pass_idx and row counter activity |

During week-3 bring-up, a HEX display that increments on every received byte tells you the UART works before the parser exists. Build the display driver early — it is 150 LEs and it pays for itself the first evening the link misbehaves.

KEY0 is asynchronous reset, synchronized to clk with a two-flop deassert. KEY1 is a manual COMPUTE trigger using whatever is already loaded, which makes a host-independent demo possible if the laptop link dies mid-presentation.

9. Resource estimate

| Block | LEs (est.) | M9K |
| --- | --- | --- |
| Config register file (12 × 32b) | 384 | 0 |
| Sequence counter + decode | 60 | 0 |
| FSM + dispatch | 50 | 0 |
| Drain shift-chain muxes (32 × 24b) | 768 | 0 |
| Result buffer control | 40 | 1 |
| Display driver | 150 | 0 |
| Total | ~1,450 | 1 |

Running total for the design so far: UART ~140, parser ~400, control ~1,450 — about 2,000 LEs, roughly 4% of the 10M50, before the lane array.

10. Verification

- Sequence trace. Assert that over one COMPUTE, weight_addr visits every value in 0..passes·R-1 exactly once, in order, and act_addr cycles correctly.

- Pipeline drain. Load a weight matrix whose last three rows are the only non-zero ones. If the FSM leaves ACC early, the result is zero. This is a directed test for the specific bug that is otherwise nearly invisible.

- Drain ordering. Load a matrix producing known distinct per-column values, confirm the shift chain deposits them in the correct result buffer addresses. Off-by-one in the shift chain reverses column order — check for exactly that.

- Multi-pass. Confirm pass 1 results do not overwrite pass 0. Use column values that make an ordering error obvious rather than random data.

- Register file. Write and read back all registers; confirm read-only registers ignore writes; confirm reset defaults match §6.3.

- TILE_CFG sweep. Run 8×8, 32×32, 48×48, 64×64 and confirm each matches the model with the correct number of rows contributing.

- NOISE_EN isolation. With all bits clear, RTL must be bit-exact against the reference model over 1,000 random matrices. This is the single most important test in the project — everything downstream assumes the clean datapath is exact.

- Busy rejection. Assert start_compute during ACC, confirm status 0x04 and no corruption of the in-flight computation.

11. Open questions for the freeze meeting

- Should CYCLE_CNT count the whole compute or just ACC? Whole compute is more honest for the report; ACC-only is the number you would quote for array throughput. Suggest recording total and computing the rest on the host.

- Reseed on every COMPUTE, or only on CTRL[2]? Explicit reseed makes runs reproducible, which matters for the model equivalence tests. Automatic reseeding makes noise statistics independent across runs, which matters for sweeps. Recommend explicit, with the host reseeding between sweep points.

- Does active_cols need to be a multiple of L? If not, the final pass drains partially and the shift chain needs a variable count. Simplest answer: round up and discard the extra columns on the host.

- Do we want a SELF_TEST command that loads a known matrix from a ROM, computes, and compares against a stored expected result — a one-packet health check for the demo? About 100 LEs and one M9K, and it makes "is the board working" answerable in one second on stage.