FeCIM: Per-Cell Physics and Parameter Derivation

Purpose: the chain from FeFET device physics to the integer arithmetic in the RTL, and the conversion formulas for turning published device numbers into register values. Owner: Lane C, with Lane D Destination: docs/device-model.md, derivation appendix

1. Notation

| Symbol | Meaning | Units |
| --- | --- | --- |
| $N$ | distinguishable conductance levels per cell | — |
| $k$ | level index, $0 \ldots N-1$ | — |
| $P_r$ | remanent polarization | µC/cm² |
| $t_{fe}$, $\varepsilon_{fe}$ | ferroelectric thickness, permittivity | nm, — |
| $V_{th}$ | threshold voltage | V |
| $MW$ | memory window, $V_{th}^{max} - V_{th}^{min}$ | V |
| $\beta$ | $\mu C_{ox} W/L$, transconductance parameter | S/V |
| $G$ | channel conductance at read bias | S |
| $V_{read}$ | gate read bias | V |
| $\sigma_{V_{th}}$ | device-to-device $V_{th}$ spread | V |
| $\nu_{ij}(t)$ | instantaneous $V_{th}$ fluctuation from trap charging | V |
| $R_w$ | bitline resistance per cell pitch | Ω |
| $M$ | rows per column | — |

2. The single-cell chain

Four transductions, each contributing one modelled non-ideality.

2.1 Pulse → switched domain fraction → polarization

HZO is polycrystalline, so coercive fields are distributed across grains. A pulse insufficient to switch every domain switches a fraction, giving intermediate net polarization:

$$P_r(k) = P_{sat}\left(\frac{2k}{N-1} - 1\right), \qquad k = 0 \ldots N-1$$

$N$ is set by how precisely the switched fraction can be controlled against the variability in that fraction. This is QUANT_LEVELS.

2.2 Polarization → threshold voltage

The bound charge at the ferroelectric/interfacial-layer boundary must be screened by the channel, shifting $V_{th}$. The ideal relation is

$$\Delta V_{th} = \frac{2 P_r t_{fe}}{\varepsilon_0 \varepsilon_{fe}}$$

which for HZO at $P_r \approx 20\ \mu\text{C/cm}^2$, $t_{fe} \approx 10$ nm, $\varepsilon_{fe} \approx 30$ predicts about 15 V. Measured memory windows are near 1 V.

Do not use the ideal expression for calibration. The order-of-magnitude gap is absorbed by the series interfacial layer, the depolarization field, and interface-trap compensation. Use the reported $MW$ directly:

$$V_{th}(k) = V_{th}^{c} - \frac{MW}{2}\left(\frac{2k}{N-1} - 1\right) + \delta_{ij} + \nu_{ij}(t)$$

The sign is negative for an n-channel device: polarization toward the substrate enhances inversion and lowers $V_{th}$.

Two perturbations enter here, and they are physically distinct:

- $\delta_{ij}$ — static, per-device, from grain orientation, size, and defect distribution. Fixed once programmed. This is D2D_SIGMA.

- $\nu_{ij}(t)$ — dynamic, changes on every read, from interface traps charging and discharging (1/f and RTN). This is READ_SIGMA.

That both originate at $V_{th}$ and differ only in timescale is why the RTL splits injection between the write path and the lane read path. Same physical quantity, two correlation times.

2.3 Threshold voltage → conductance

In the linear region at small $V_{ds}$:

$$I_d = \beta\left(V_{read} - V_{th} - \frac{V_{ds}}{2}\right)V_{ds} \quad\Longrightarrow\quad G = \frac{\partial I_d}{\partial V_{ds}} \approx \beta\left(V_{read} - V_{th}\right)$$

So $G$ is linear in $V_{th}$, and $V_{th}$ is linear in polarization — which is the entire reason the crossbar computes a multiply at all. The full conductance range is

$$G_{max} - G_{min} = \beta \cdot MW$$

The $-V_{ds}/2$ term is the nonlinearity the model currently ignores; see §6.2.

2.4 Stuck cells

Pinned domains, dead grains, and gate-stack defects give cells that ignore programming:

$$G_{ij} = \begin{cases} G_{min} \text{ or } G_{max} & \text{with probability } p_{stuck} \ G(k) & \text{otherwise} \end{cases}$$

This is STUCK_RATE.

3. The column sum

Ideally, by KCL on the bitline:

$$I_j = \sum_{i=0}^{M-1} G_{ij} V_i$$

Note that no DC current flows in the gate lines — a FeFET gate is capacitive, so wordline IR drop is a settling-time issue, not a static error. The DC drop is on the bitline, where current accumulates.

Cell $i$'s current traverses the bitline segments between it and the sense amplifier, so the source-node voltage is elevated by the accumulated current through those segments:

$$\Delta V_i = R_w \sum_{i' } c(i,i'), I_{i'}$$

This is a coupled problem — the drop depends on every cell's current, which depends on the data. The modelled approximation replaces the actual currents with a nominal $\bar{I}$ and keeps only row-position dependence:

$$\alpha_i \approx 1 - \frac{R_w \bar{I}}{V_{ds}} f(i), \qquad f(i) \approx \frac{i(2M-i+1)}{2}$$

$$I_j \approx \sum_i \alpha_i G_{ij} V_i$$

This is atten_rom. The factorization in MAC array spec §8 — applying $\alpha_i$ to the activation once rather than to each product — is valid precisely because $\alpha$ was reduced to a function of $i$ alone. A rigorous treatment is input-dependent and breaks it.

Then the ADC:

$$D_j = Q_b(I_j), \qquad b = \texttt{ADC_BITS}$$

4. Correspondence to the RTL

Normalized weight $w \in [-128, 127]$ maps to conductance by

$$G_{ij} = G_{min} + \left(G_{max} - G_{min}\right)\frac{u_q}{255}, \qquad u_q = w_q + 128$$

so one weight LSB corresponds to $(G_{max}-G_{min})/255 = \beta, MW/255$, and equivalently to a $V_{th}$ shift of $MW/255$.

That last equivalence is the key calibration identity:

$$\boxed{;\Delta w ,[\text{LSB}] = \frac{255}{MW},\Delta V_{th},[\text{V}];}$$

4.1 Quantization

| Physics | RTL |
| --- | --- |
| $k = \text{round}!\left(\frac{u}{255}(N-1)\right)$ | level = (u * QUANT_LEVELS) >> 8 |
| $G(k)$, re-expressed in weight LSBs | u_q = (level * QUANT_MULT) >> 8 |

$$\texttt{QUANT_LEVELS} = N, \qquad \texttt{QUANT_MULT} = \text{round}!\left(\frac{255 \cdot 256}{N-1}\right)$$

Check, $N = 8$: QUANT_MULT = 9326. For $u = 255$: level = 7, u_q = 255. For level = 3: u_q = 109, against $3 \times 255/7 = 109.3$. ✓

4.2 Device-to-device variation

The RTL forms a triangular variate from two hash bytes:

ih_sum = h_d2d[7:0] + h_d2d[15:8]        // 0..510

ih9    = ih_sum - 255                    // ±255,  σ = 104.5

d2d    = (ih9 * D2D_SIGMA) >> 8

Each byte is uniform on $[0,255]$ with variance $(256^2-1)/12 = 5461$, so $\sigma_{ih9} = \sqrt{2 \times 5461} = 104.5$. Therefore $\sigma_{d2d} = 104.5 \cdot \texttt{D2D_SIGMA}/256$, giving

$$\boxed{;\texttt{D2D_SIGMA} = \text{round}!\left(2.450 \cdot \sigma_w\right);} \qquad \sigma_w = \frac{255,\sigma_{V_{th}}}{MW}$$

Worked example. A paper reports $\sigma_{V_{th}} = 50$ mV and $MW = 1.0$ V:

$$\sigma_w = 255 \times 0.05 / 1.0 = 12.8 \text{ LSB} \quad\Longrightarrow\quad \texttt{D2D_SIGMA} = \text{round}(2.450 \times 12.8) = 31$$

Maximum representable: $\texttt{D2D_SIGMA} = 255 \Rightarrow \sigma_w = 104$ LSB, about 41% of full scale. Ample.

4.3 Read noise

Four LFSR bytes, halved to fit 9 bits:

ih_sum = lfsr[7:0]+lfsr[15:8]+lfsr[23:16]+lfsr[31:24]   // 0..1020

ih_c   = ih_sum - 510                                    // ±510, σ = 147.8

ih9    = ih_c >>> 1                                      // ±255, σ = 73.9

noise  = (ih9 * READ_SIGMA) >>> 9

$\sigma_{noise} = 73.9 \cdot \texttt{READ_SIGMA}/512 = 0.1443,\texttt{READ_SIGMA}$, so

$$\boxed{;\texttt{READ_SIGMA} = \text{round}!\left(6.93 \cdot \sigma_w\right);}$$

Maximum: $\texttt{READ_SIGMA} = 255 \Rightarrow \sigma_w = 36.8$ LSB, which sits at $127/36.8 = 3.4\sigma$ from the clamp — so clipping is negligible even at maximum setting. Realistic read noise is a few LSB, so the range is roughly ten times what is needed.

4.4 Stuck cells

$$\texttt{STUCK_RATE} = \text{round}!\left(65536 \cdot p_{stuck}\right)$$

Resolution is $1/65536 = 0.0015%$.

4.5 IR drop

$$\texttt{atten_rom}[i] = \text{round}!\left(256,\alpha_i\right)$$

A first-order linear model, adequate for a parameter sweep, is $\alpha_i \approx 1 - \kappa, i/M$ with $\kappa$ the worst-case fractional drop at the far row. Sweep $\kappa$; it is not a device parameter and should not be presented as one.

5. Full parameter summary

| Register | Physical origin | Conversion |
| --- | --- | --- |
| QUANT_LEVELS | controllable switched-domain fraction | $N$ directly |
| QUANT_MULT | — (arithmetic) | $\text{round}(255\cdot256/(N-1))$ |
| D2D_SIGMA | grain statistics → static $V_{th}$ spread | $2.450 \cdot 255\sigma_{V_{th}}/MW$ |
| READ_SIGMA | interface traps → dynamic $V_{th}$ fluctuation | $6.93 \cdot 255\sigma_{V_{th},rms}/MW$ |
| STUCK_RATE | pinned domains, gate-stack defects | $65536,p_{stuck}$ |
| atten_rom[i] | bitline resistance (circuit, not device) | $256,\alpha_i$ |
| ADC_BITS | converter design choice | $b$ directly |
| retention factor | depolarization field, trap compensation | $256,P_r(t)/P_r(0)$ |

Only the first five are device properties. atten_rom and ADC_BITS are circuit design parameters and should be labelled as such in the confidence column — presenting them as measured device data would be misleading.

6. Two problems this derivation found

6.1 The ADC shift is wrong

MAC array spec §9 computes sh = ACC_W - adc_bits with ACC_W = 32. But the accumulator's actual range is

$$254 \times 255 \times 128 = 8{,}290{,}560 \approx 2^{23}$$

so real values occupy 24 signed bits, not 32. With $b = 8$, the spec gives sh = 24, quantizing over $\pm 2^{31}$ — a range the signal never approaches. Every result would collapse into one or two ADC codes. The ADC would appear catastrophically destructive at any setting, and the obvious suspicion would fall on the rounding logic rather than the full-scale reference.

Fix: quantize over the used range, not the container width. Add to the package

parameter int ACC_USED_W = $clog2(ACC_MAX_MAG) + 1;   // 24 at 128 rows

and use sh = ACC_USED_W - adc_bits. Better still, make ADC_SHIFT a host-written register computed from the actual expected accumulator range for the loaded network — real ADC full-scale is a design choice, and exposing it makes that explicit and sweepable.

Add a verification case: with adc_bits = ACC_USED_W, the ADC must be an exact no-op.

6.2 READ_SIGMA scaling is documented inconsistently

MAC array spec §5.3 describes sigma9 as Q1.8 spanning 0 to 0.996, but the code shifts by 9, making the effective range 0 to 0.498. The text and the arithmetic disagree.

Keep the shift at 9 — it is the better choice, since it places maximum noise at 3.4σ from the clamp instead of 1.7σ, avoiding clip distortion at high settings. Correct the prose instead: READ_SIGMA is sigma9/512, not Q1.8. The conversion in §4.3 above assumes the shift of 9.

This matters beyond documentation: model_exact must use the same shift, and a factor-of-two disagreement between model and RTL would fail L1 equivalence with no obvious cause.

7. Known gap: I–V nonlinearity

The model assumes an exact integer multiply, which corresponds to a linearized read scheme — either pulse-width-encoded activations at fixed $V_{ds}$, or a series resistor dominating the cell conductance as in the 1FeFET-1R construction.

If activations are instead encoded as $V_{ds}$ amplitude, the $-V_{ds}/2$ term in §2.3 makes the product nonlinear in the activation, and driving the gate harder eventually leaves the linear region entirely.

State this as a known gap in the report, since NeuroSim and AIHWKit both model it. It is a strong semester-2 addition and cheap: a lookup table on the activation broadcast path, one shared instance, structurally identical to atten_rom. Adding it would also let the tool answer a question the linearized assumption hides — how much accuracy is bought by the series resistor in a 1FeFET-1R cell.