FeCIM: Model and Verification Design

Owner: Lane C Consumers: Lanes A and B (RTL oracle), Lane D (sim backend and sweeps), the report Milestone: model core weeks 2–5, calibration week 7, coverage complete week 10 Revision: 2 — calibration reworked for published FeFET parameter sources

1. The structural decision: two models, not one

The most common way this goes wrong is writing one "reference model," discovering it does not match the RTL, and being unable to tell whether the RTL is buggy or the model is.

There are actually two different questions, and they need two different artifacts:

|  | model_ideal | model_exact |
| --- | --- | --- |
| Question it answers | What should a perfect crossbar compute? | What does this hardware compute? |
| Arithmetic | float64 / int64, NumPy | integer, mirrors RTL bit for bit |
| Non-idealities | none | all of them |
| Speed | fast, vectorized | slower, cycle-faithful |
| Role | validates model_exact in the clean case | oracle for the RTL |

The verification chain is two links, not one:

NumPy ideal  ──(clean case only)──►  model_exact  ──(every case)──►  RTL

model_ideal cannot check the noisy cases — it has no notion of noise. model_exact cannot check itself. Together they pin down both ends: if RTL and model_exact disagree, one has a bug; if they agree but model_ideal disagrees in the clean case, the bug is in model_exact's arithmetic.

2. Bit-exactness constrains how the model is written

model_exact must reproduce the RTL exactly, not approximately. That forbids things that would otherwise be natural:

- No floating point anywhere in the datapath. Q1.8 scaling must be an integer multiply and arithmetic shift matching the RTL's >>> 9, not a float multiply and round.

- The hash must be identical, including overflow. Use uint16_t so the 16-bit rounds wrap the way the RTL does; a wider type silently changes results.

- The LFSR advances on the same events — on accumulate, not on every step. Model a clock-gated LFSR, not a random number generator.

- Clamp where the RTL clamps. Read noise clamps at ±127; the write-path add saturates at ±127/−128; the lane add does not saturate because 9 bits cover the range. Adding a "safety" clamp the RTL does not have breaks equivalence.

- Rounding direction must match. ADC round-to-nearest is (x + half) >> sh, not round(x / 2^sh), which differs on negative halves.

- Distribution shapes must match the construction, not the intent. Read noise sums four LFSR bytes; write-path variation sums two hash bytes and is triangular. The model reproduces those exact sums, not an idealized Gaussian.

2.1 One thing that is free

Integer addition is associative and commutative, so the RTL's sequential row-by-row accumulation and NumPy's matmul produce identical sums regardless of order.

Worth appreciating: had the datapath been floating point, accumulation order would matter and bit-exact comparison against a vectorized reference would be impossible. The entire verification strategy rests on the design being integer.

3. The equivalence hierarchy

Five levels, in dependency order. Each is a different kind of claim needing a different kind of test.

| Level | Claim | Type | Gate |
| --- | --- | --- | --- |
| L0 | Clean RTL ≡ clean model_exact ≡ model_ideal | exact | week 4 |
| L1 | Each non-ideality alone: RTL ≡ model_exact | exact | week 5 |
| L2 | All non-idealities together: RTL ≡ model_exact | exact | week 5 |
| L3 | model_exact statistics match published device behaviour | statistical | week 7 |
| L4 | Hardware ≡ model_exact at matched seed | exact | week 5 |

L3 is categorically different and must not be conflated with the others. L0–L2 and L4 are bit-exactness claims about whether the implementation is correct. L3 asks whether the implementation reproduces the physics the literature reports — measured mean, variance, and fault rate against published values. It can only fail by being unphysical, never by being off by one.

Keeping them separate matters because they fail differently and different people fix them. An L2 failure is an RTL bug. An L3 failure is a calibration problem. Reporting them in the same suite invites confusion about which is which.

3.1 L0 is the load-bearing gate

Nothing downstream means anything until clean RTL matches clean model exactly over thousands of random matrices. Run it in CI on every commit. If it goes red, stop feature work.

4. Model architecture

4.1 One implementation, three consumers

┌──────────────────────┐

│   model_exact (C++)  │

└──┬────────┬───────┬──┘

│        │       │

┌──────────────┘        │       └──────────────┐

▼                       ▼                      ▼

Verilator co-sim        pybind11 module        calibration tools

(RTL oracle)            (sim backend)          (parameter fitting)

The sim backend must be the same code that verified the RTL. An independent Python reimplementation could drift, and then the "universal build target" would be running something never validated against hardware.

4.2 Packaging maps onto M0's build requirement

M0 asks for a source distribution buildable with a single command and binary distributions for supported platforms. The C++/pybind11 setup satisfies both directly:

- Source distribution: pip install . compiles the extension. Needs a C++17 compiler — present by default on Ubuntu and macOS, needs MSVC Build Tools on Windows. Document this prominently; it is the one friction point.

- Binary distribution: cibuildwheel in GitHub Actions produces wheels for Windows, Ubuntu, and macOS on every tagged release.

Keep the C++ core dependency-free (C++17 standard library only). pybind11 is header-only and pip-installable. Anything more — Eigen, Boost — turns a one-command build into a support ticket.

5. Co-simulation harness

The highest-value infrastructure Lane C builds, and the reason Verilator was chosen over Icarus. Verilator compiles the RTL to C++, so model and DUT run in one process with full visibility into internal signals:

for (int cycle = 0; cycle < n_cycles; cycle++) {

dut->clk = 0; dut->eval();

dut->clk = 1; dut->eval();

model.step();

for (int l = 0; l < NUM_LANES; l++) {

if (dut->rootp->fecim_top__DOT__acc[l] != model.acc[l]) {

report_mismatch(cycle, l, dut_value, model_value);

return FAIL;

}

}

}

The failure message says "lane 17 diverged at cycle 412," not "the answer is wrong." That difference is worth days over a semester.

Compare at three depths, in this order — the first mismatch upstream explains all the ones downstream:

- w_n (post-noise weight) per lane

- prod per lane

- acc per lane

5.1 Virtual UART

For integration tests, drive the Verilated top level through the same byte stream the real board would receive. Lane D's packet encoder is then tested against the actual RTL parser in CI before hardware exists — which is how the week-3 bring-up becomes a wiring exercise rather than a protocol debugging session.

6. Coverage without coverage tools

Questa Intel Starter Edition has no functional coverage, so coverage is defined and tracked manually — as an explicit matrix in the repo, not as a feeling about how much testing has happened.

6.1 The verification matrix

| Config | Directed | Random ×1000 | Corner | Hardware |
| --- | --- | --- | --- | --- |
| Clean, 128×128 | ✓ | ✓ | ✓ | ✓ |
| Quant only, N ∈ {2,3,5,8,255} | ✓ | ✓ | ✓ | ✓ |
| D2D only, σ ∈ {0, 8, 32, 255} | ✓ | ✓ | ✓ | ✓ |
| Read noise only | ✓ | ✓ | ✓ | ✓ |
| Stuck only, rate ∈ {0, 1%, 10%} | ✓ | ✓ | ✓ | ✓ |
| ADC only, bits ∈ {4, 8, 16, 32} | ✓ | ✓ | ✓ | ✓ |
| IR only | ✓ | ✓ | ✓ | ✓ |
| All enabled | ✓ | ✓ | ✓ | ✓ |
| Tile 8×8, 32×32, 48×48, 128×128 | ✓ | ✓ | — | ✓ |

Non-power-of-two quantization levels (3, 5) and non-multiple-of-64 tile widths (48) are there deliberately — they exercise the reciprocal path and the argmax mask that powers of two would hide.

6.2 Corner cases, enumerated

Random stimulus will not find these:

- All weights +127, all activations 255, maximum positive noise — accumulator headroom

- Same with −128 — negative overflow and sign handling

- All weights zero — argmax tie across every column

- Exactly two columns tied at maximum — tie-break rule

- One-hot activation at row 127 — last-row pipeline inclusion

- One-hot activation at row 0 — first-row clear-versus-accumulate priority

- stuck_rate = 0xFFFF — every cell stuck

- QUANT_LEVELS = 2 — binary weights

- adc_bits = ACC_W — ADC enabled but lossless, must be an exact no-op

- Maximum read noise with weights at ±127 — assert w_n never exceeds 9 bits signed

The "enabled but lossless" case is a good bug detector: a path that changes the answer when it should not means the shift arithmetic is wrong.

7. Calibration: from published measurements to register values

This is a different activity from everything above. Verification asks "is the implementation correct." Calibration asks "are the numbers right."

7.1 The source change, and what it buys

Parameters come from published FeFET device characterization, not in-house capacitor measurements. Three consequences worth stating plainly in the report:

The polarization-to-conductance bridge is gone. In-house P–E data on capacitors would have required assuming polarization state → threshold voltage shift → channel conductance. That was the weakest link in the chain. Published FeFET work reports conductance or current directly, so the assumption does not arise. This is a rigor improvement, not a compromise.

One process becomes several. A single lab's data gives one point. Published data gives a distribution across fabs, nodes, and programming schemes — which enables a question a single-device study cannot answer: how much does device variability alone move network accuracy?

Everything is reproducible. Every parameter traces to a citation, so any reader can rerun the study.

7.2 The chain, per parameter

Every register value gets a documented derivation in docs/device-model.md:

| Register | Extract from | Chain | Typical confidence |
| --- | --- | --- | --- |
| QUANT_LEVELS | reported multilevel states | distinguishable conductance levels, usually stated directly | direct |
| QUANT_MULT | computed from QUANT_LEVELS | host arithmetic, no measurement | exact |
| D2D_SIGMA | device-to-device spread | conductance variance across cells | direct |
| READ_SIGMA | cycle-to-cycle variation | repeated-read spread on one cell | direct |
| STUCK_RATE | yield / failed-bit data | fraction of non-programmable cells | varies — often under-reported |
| ADC_BITS | peripheral circuit assumptions | design choice, not a device property | design parameter |
| atten_rom | wire resistance and array size | circuit-level estimate | estimated |
| retention factor (sem. 2) | retention measurements vs time | state decay toward depolarized | direct |

Mark the confidence column honestly. A reviewer who sees "estimated" next to atten_rom will trust the "direct" entries more, not less. Presenting every parameter with equal authority undersells the ones that are genuinely well-sourced.

7.3 Provenance is the new load-bearing assumption

Every calibration rests on one. With public data it is parameter-set provenance.

Published parameters come from different papers, processes, nodes, and measurement conditions. Taking QUANT_LEVELS from one source and D2D_SIGMA from another silently assumes they describe the same device — which is frequently false and always invisible in the final number.

The rule:

- Each complete parameter set traces to a single source where possible. Name it.

- Composite sets are labelled composite, with each parameter's source listed, and are not presented as characterizing a real device.

- A parameter with no usable published value is swept, not guessed (§7.5).

docs/device-model.md is structured as one table per parameter set, with columns for parameter, value, source citation, and confidence. That table is what a skeptical reader will actually check.

7.4 Target: three parameter sets

Aim for three complete sets from distinct processes. The comparison across them is the result; any one of them alone is just a configuration.

Assign specific papers to specific people in week 1 and record it in the issue tracker. Finding usable sets is a search problem with uncertain completion time and no hardware dependency, so it should run in the background from the start rather than becoming a week-7 scramble.

What makes a source usable: it reports enough parameters to fill most of a set, states measurement conditions, and gives spread rather than only a mean. A paper reporting a single "8 levels achieved" with no variance data is a partial source at best.

7.5 Sweep what the literature does not pin down

Where a parameter is poorly reported — STUCK_RATE especially, since yield data is often omitted — do not pick a point estimate. Sweep it across a plausible range and present a band.

"Accuracy stays above 90% for stuck-cell rates below 2%" is a stronger and more honest claim than "accuracy is 93.2% at our assumed fault rate," and it survives someone disagreeing with the assumption.

8. Reproducibility

Every result in the report must be regenerable from the repo:

- Seeds recorded, never implicit. Every sweep writes its seed set to the results file. A plot that cannot be regenerated is not evidence.

- Golden fixtures in the repo, not generated at test time. tb/golden/ holds byte sequences with hand-computed CRCs, shared between the C++ testbench and the Python driver tests, so encoding drift fails immediately in both places.

- Parameter sets are version-controlled data files, not values typed into a script. One file per source, with the citation in the file.

- Results carry provenance. Each sweep output records git commit, RTL build ID, the IDENTIFY string from the board, the parameter set used, and full config.

- CI runs L0 on every push. Feature work stops if it goes red.

With public data, reproducibility stops being merely good practice and becomes the project's main claim to credibility — a reader with no special access can rerun everything.

9. Lane C schedule

| Week | Deliverable |
| --- | --- |
| 1 | C++ skeleton, quantization only, unit test harness, CI green. Literature survey begins. |
| 2 | model_ideal clean MVM; model_exact clean path; L0 in simulation |
| 3 | Hash, LFSR, all non-ideality math; candidate parameter sources shortlisted |
| 4 | Verilator co-sim harness; L0 gate on real RTL; virtual UART |
| 5 | L1 and L2 across the matrix; L4 on hardware |
| 6 | Regression suite stable; supports the first end-to-end demo |
| 7 | First parameter set complete; docs/device-model.md with provenance and confidence |
| 8 | Matrix rerun at 128×128 / 64 lanes |
| 9 | L3 statistical validation; uncertainty bands for sweeps |
| 10 | Coverage matrix closed; verification report drafted |
| 11 | Verification report final, including known gaps |

Weeks 4 and 7 are the hard gates: the co-simulation harness and the first documented parameter set.

10. Open questions

- Cycle-accurate or transaction-accurate model_exact? Cycle-accurate makes the co-sim comparison per-cycle and localizes bugs precisely; transaction-accurate is faster for sweeps. Recommend cycle-accurate core with a fast path that skips intermediate state when only the final result is needed.

- Should the sim backend report a modelled cycle count so throughput plots work without a board? Cheap, and it makes the demo fallback more complete.

- How many published sources are realistically available with enough parameters to fill a complete set? This determines whether the three-process comparison is achievable. Answer it in weeks 1–3, not week 7 — and if only one usable set exists, the semester-2 plan needs to change while there is still time.

- Do any sources report level-dependent variation? Larger spread at intermediate states is physically expected, since those rely on partial domain switching. If a source reports it, modelling it costs one small lookup table and is a genuinely distinctive result.