FeCIM Component Spec: Packet Parser and Response Generator

Module: packet_parser, packet_tx, crc8 Owner: Lane A (RTL), protocol frozen jointly with Lane C Depends on: uart_rx / uart_tx (byte layer) Milestone: Week 4 Status: draft — freeze in week 1, before Lane C writes the driver

1. Purpose and scope

Turns the byte stream from uart_rx into validated commands, and turns command results back into a byte stream for uart_tx. Owns framing, length checking, CRC, timeout recovery, and dispatch.

Does not own: the meaning of any command's payload beyond routing it, the config register map's semantics, or anything about the MAC array.

This is the seam between all three work lanes. Lane C writes the host encoder against this document, Lane B writes the equivalence tests against it, and Lane A implements it. Changing it after week 1 costs three people a week.

2. Frame format

Host → FPGA:   [0xA5][CMD:1][LEN:2][PAYLOAD:LEN][CRC:1]

FPGA → Host:   [0x5A][STATUS:1][LEN:2][PAYLOAD:LEN][CRC:1]

- Sync — 0xA5 inbound, 0x5A outbound. Distinct values make direction obvious on a logic analyzer. 0xA5 = 10100101, chosen for its alternating pattern: it is unlikely to be produced by a stuck line or a half-shifted byte.

- LEN — payload length in bytes, little-endian, excludes header and CRC.

- CRC — CRC-8 over CMD, LEN, and PAYLOAD. The sync byte is excluded (it is a delimiter, not content).

2.1 Byte order

Everything multi-byte is little-endian, at every layer: LEN, config values, weight addresses, result words. This matches the x86 host and Python's struct '<' prefix, so no host-side swapping is ever needed.

Write this on the whiteboard. Endianness disagreement between the RTL and the driver is the single most common week-4 integration bug, and it presents as plausible-looking garbage rather than an obvious failure.

2.2 Size limits

parameter MAX_PAYLOAD = 512;   // bytes

Any packet declaring LEN > MAX_PAYLOAD is rejected immediately at the LEN1 state, without waiting for the payload. LEN stays 16 bits for protocol headroom even though the hardware enforces 512.

512 is chosen so a 128-column result readout (128 × 3 = 384 bytes) fits in one packet. Weight transfers should use 256-byte chunks — see §4.2.

3. Command set

| CMD | Name | Payload | Response payload | Class |
| --- | --- | --- | --- | --- |
| 0x01 | WRITE_WEIGHTS | [addr:2][data:N] | — | bulk |
| 0x02 | WRITE_ACT | [addr:2][data:N] | — | bulk |
| 0x03 | SET_CONFIG | [reg:1][value:4] | — | control |
| 0x04 | COMPUTE | — | — | control |
| 0x05 | READ_RESULT | [start:2][count:2] | count × 3 bytes | control |
| 0x06 | IDENTIFY | — | 16 bytes (see §3.1) | control |
| 0x07 | GET_CONFIG | [reg:1] | [value:4] | control |

3.1 IDENTIFY response

Sixteen bytes, fixed layout, little-endian:

| Offset | Width | Field |
| --- | --- | --- |
| 0 | 2 | Magic 0xFEC1 |
| 2 | 1 | Protocol version |
| 3 | 1 | RTL build ID |
| 4 | 2 | NUM_LANES |
| 6 | 2 | TILE_ROWS |
| 8 | 2 | TILE_COLS |
| 10 | 2 | Framing error count (from uart_rx) |
| 12 | 2 | CRC error count |
| 14 | 2 | Timeout count |

The driver calls this on connect and refuses to proceed on a version mismatch, which turns "the numbers are wrong" into "your bitstream is stale" — a much faster diagnosis. The three error counters make the week-3 bring-up debuggable without a scope: if CRC errors climb while framing errors stay at zero, the problem is your parser, not your baud rate.

4. Payload handling: two classes

The core design decision. A 4 KB weight tile cannot be buffered on-chip before validation, but a corrupted config write must never take effect. These need different treatment, and conflating them produces either a memory-hungry design or a silent failure mode.

4.1 Control commands — buffer and commit

Payload ≤ 8 bytes, held in a small register buffer. No effect occurs until CRC passes. A corrupted SET_CONFIG is discarded whole; the host sees an error status and retries.

This matters because config corruption is silent. A wrong noise sigma produces plausible numbers that are simply wrong, and you would not discover it until a sweep looked strange three weeks later.

4.2 Bulk commands — stream through

Payload is [dest_addr:2] followed by data bytes. The address is captured, then each subsequent byte is emitted on the streaming interface with an auto-incrementing address as it arrives. Nothing is buffered.

CRC failure means the destination has already been partially written with bad data. That is acceptable, and deliberately so: weight and activation writes are idempotent, so the host simply re-sends the chunk and the correct values overwrite the corrupt ones.

This is why bulk transfers are chunked at 256 bytes rather than sent as one 4 KB packet. A CRC failure costs a 256-byte retry instead of a 4 KB one, and the host can retry a single chunk without rebuilding the whole tile.

4 KB tile (64×64 × 8b) = 16 chunks × 256 bytes

Per chunk: [0xA5][0x01][0x02 0x01][addr:2][data:256][crc]

5. packet_parser state machine

┌──────────────────────────────────────────────┐

│                                              │

▼                                              │

┌──────┐  rx=0xA5   ┌─────┐        ┌──────┐       │

│ HUNT │───────────►│ CMD │───────►│ LEN0 │       │

└──────┘            └─────┘        └──────┘       │

▲                                   │          │

│                                   ▼          │

│                               ┌──────┐       │

│            LEN>MAX_PAYLOAD    │ LEN1 │       │

│            ◄──────────────────└──────┘       │

│                                   │          │

│                                   ▼          │

│                             ┌──────────┐     │

│                             │ PAYLOAD  │◄─┐  │

│                             └──────────┘  │  │

│                                   │       │  │

│                          count==LEN       │  │

│                                   ▼       │  │

│                               ┌─────┐     │  │

│                               │ CRC │     │  │

│                               └─────┘     │  │

│                                   │       │  │

│      CRC fail / timeout           ▼       │  │

└───────────────────────────┌──────────┐    │  │

│ DISPATCH │    │  │

└──────────┘    │  │

│         │  │

▼         │  │

┌──────────┐    │  │

│ RESPOND  │────┴──┘

└──────────┘

Any state except HUNT can be aborted by an inter-byte timeout, returning to HUNT.

5.1 Timeout — do not skip this

Without it, a LEN field corrupted from 0x0100 to 0xFF00 leaves the parser waiting for 65,280 bytes that will never arrive. The link hangs and only a board reset recovers it. Since LEN is covered by CRC but consumed before CRC is checked, this is not a hypothetical.

parameter TIMEOUT_CYCLES = 24'd5_000_000;   // 100 ms at 50 MHz

The counter resets on every received byte and only runs outside HUNT. On expiry: increment the timeout counter, return to HUNT, send no response. The host's own read timeout fires and it retries.

100 ms is ~1,150 byte-times at 115200 — generous enough never to trigger on a healthy link, short enough that recovery is invisible to a human operator.

5.2 Resynchronization

0xA5 will appear inside payload data — it is a valid weight value, and roughly one byte in 256 will be one. Sync detection alone is therefore not sufficient to find frame boundaries.

Recovery relies on the combination: a false sync produces a packet whose CRC fails (probability 255/256) or whose length triggers a timeout, and the parser returns to HUNT and tries again. This converges quickly in practice.

The host cooperates by waiting one full timeout period after any error before retransmitting. This guarantees the parser is back in HUNT and the line is quiet, rather than the host racing to send a new packet into a parser still consuming the previous one's phantom payload. Document this in the driver — it is a protocol requirement, not an implementation detail.

6. CRC-8

Polynomial 0x07 (x⁸ + x² + x + 1, CRC-8/ATM), init 0x00, no reflection, no final XOR. Computed byte-serially as each byte arrives, so no buffering is needed and the check completes one cycle after the CRC byte.

function [7:0] crc8_byte(input [7:0] crc, input [7:0] data);

integer i;

reg [7:0] c;

begin

c = crc ^ data;

for (i = 0; i < 8; i = i + 1)

c = c[7] ? ((c << 1) ^ 8'h07) : (c << 1);

crc8_byte = c;

end

endfunction

Unrolled to combinational logic, this is an XOR tree of roughly 30 LEs.

Why CRC-8 is enough here. The dominant error mode on a 15 cm cable at 115200 is not line noise, it is parser desynchronization — and a desynced frame fails CRC-8 with probability 255/256, same as any stronger polynomial. For genuine bit errors, CRC-8 catches all single-bit errors and all bursts up to 8 bits, which covers realistic UART failure modes.

If you later run at 921600 over a longer cable and see CRC errors in the IDENTIFY counters, upgrade to CRC-16-CCITT (poly 0x1021, init 0xFFFF) — about 20 more LEs and one more byte per frame. Design the driver so this is a one-line change.

7. Interfaces

7.1 packet_parser ports

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk, rst_n | in | 1 |  |
| rx_data | in | 8 | from uart_rx |
| rx_valid | in | 1 |  |
| Bulk stream |  |  |  |
| bulk_we | out | 1 | one cycle per payload byte |
| bulk_target | out | 2 | 0=weights, 1=activations |
| bulk_addr | out | 16 | auto-incrementing |
| bulk_data | out | 8 |  |
| Config |  |  |  |
| cfg_we | out | 1 | pulse, after CRC passes |
| cfg_addr | out | 8 |  |
| cfg_wdata | out | 32 |  |
| cfg_rdata | in | 32 | for GET_CONFIG |
| Control |  |  |  |
| start_compute | out | 1 | pulse |
| compute_busy | in | 1 |  |
| Readout |  |  |  |
| rd_addr | out | 16 |  |
| rd_data | in | 24 | combinational or 1-cycle |
| Response |  |  |  |
| resp_* | — | — | handshake to packet_tx |

7.2 Status codes

| Code | Meaning |
| --- | --- |
| 0x00 | OK |
| 0x01 | CRC error |
| 0x02 | Bad length (> MAX_PAYLOAD, or wrong for this command) |
| 0x03 | Unknown command |
| 0x04 | Busy (compute_busy asserted) |
| 0x05 | Address out of range |

Every command produces a response, including errors — except timeouts, which produce silence by definition. This keeps the driver a simple request/response loop: send, wait for a response or a read timeout, retry on either.

7.3 Busy handling

compute_busy is asserted for ~135 cycles (2.7 µs), while the next packet cannot arrive for at least 86 µs at 115200. The busy path will never be exercised in normal operation. Implement it anyway — it costs one state and one status code, and it stops being unreachable the moment someone raises the baud rate or adds a streaming mode.

8. Resource estimate

| Block | LEs (est.) |
| --- | --- |
| CRC-8 combinational | 30 |
| Byte / length counters | 55 |
| Control payload buffer (8 B) | 70 |
| Parser FSM + dispatch | 60 |
| Timeout counter (24 b) | 50 |
| packet_tx FSM + mux | 80 |
| Error counters (3 × 16 b) | 55 |
| Total | ~400 |

Zero M9K, zero multipliers. Roughly 0.8% of the 10M50. Combined with the UART at ~140 LEs, the entire host interface is about 1% of the chip.

9. Verification

Lane B owns 1–6; Lane A owns 7–8. All of these run against the C++ reference model as well, since the model must accept the same packets for the sim backend to be a genuine drop-in.

- Golden packets. A fixture file of hand-computed byte sequences with known-correct CRCs, shared between the RTL testbench and the Python driver tests. Both sides must agree byte-for-byte. This catches endianness disagreement immediately.

- CRC vector check. RTL CRC-8 against a Python crcmod reference over 10,000 random payloads.

- Every command. Correct dispatch, correct response, correct status for each of the seven commands.

- Corruption sweep. For each byte position in a valid packet, flip one bit and confirm the parser rejects and recovers. Includes bit flips in LEN, which is the case that exercises the timeout path.

- Timeout. Send a truncated packet, confirm the parser returns to HUNT after TIMEOUT_CYCLES and correctly accepts the next valid packet.

- False sync. Send a bulk payload containing 0xA5 bytes and confirm the parser does not resynchronize mid-packet.

- Bulk streaming. Write a full 4 KB tile in 16 chunks; read it back and compare. Then deliberately corrupt chunk 7 in transit, confirm the error status, retry that chunk alone, and confirm the tile is correct.

- Soak. 100,000 random packets over hardware with error counters checked at the end. Run this overnight in week 4 — it finds the once-per-50,000 synchronizer bug that no directed test will.

10. Open questions for the freeze meeting

- MAX_PAYLOAD of 512 or 256? 512 lets a 128-column readout come back in one packet; 256 keeps buffers smaller and retries cheaper. Recommend 512, with the convention that writes chunk at 256 anyway.

- Should GET_CONFIG exist? It is ~20 LEs and makes the driver able to verify what it set, which is worth it during calibration when you are sweeping parameters and need to be certain the board agrees with the host.

- Sequence numbers? A one-byte counter would let the host detect a dropped response rather than inferring it from a timeout. Probably unnecessary given request/response lockstep, but decide now — retrofitting changes the frame.

- Does COMPUTE respond immediately or on completion? Respond on completion. At 2.7 µs the host cannot observe the difference, and it makes the driver's compute() naturally blocking with no polling loop.