FeCIM RTL Conventions

Language: SystemVerilog (IEEE 1800), synthesizable subset Applies to: all files under rtl/ and tb/ Owner: Lane A, with Lane B on the testbench side Status: amends the UART, packet parser, and control FSM specs

1. Toolchain support, and the constraint it imposes

Quartus Prime Lite synthesizes a subset of SystemVerilog. The subset is generous enough for a design this size, but it is worth knowing where the edges are before someone spends an afternoon on a construct that will not fit.

1.1 Use freely

| Construct | Notes |
| --- | --- |
| logic | replaces both reg and wire; use everywhere |
| always_ff / always_comb | intent-declaring; Quartus checks them |
| typedef enum | for all state machines and protocol codes |
| packed struct | for bit-field registers |
| package / import | the single-source-of-truth mechanism — see §3 |
| unique case / priority case | synthesis hints plus simulation assertions |
| int, bit, $clog2 | parameter arithmetic |
| '0, '1, 'x | width-agnostic fills |
| .* and .name port connections | fewer positional-connection bugs |
| for (int i = ...) in generate | lane array instantiation |

1.2 Avoid

| Construct | Reason |
| --- | --- |
| interface / modport | supported but historically inconsistent in Lite; a design this small does not need it, and a synthesis surprise in week 4 is expensive |
| unpacked struct on ports | poor synthesis support |
| always_latch | you do not want latches; if one appears it is a bug |
| classes, rand, constrained random | simulation only, and see §2.2 on tool limits |
| reg / wire | not wrong, but mixing them with logic defeats the point |

1.3 Required style

- always_ff for every sequential block, always_comb for every combinational one. Never bare always. always_comb errors on incomplete assignment, which turns accidental latches into compile failures instead of silent hardware.

- Async assert, sync deassert reset: always_ff @(posedge clk or negedge rst_n).

- One always_ff block may drive a given signal. logic enforces this — a second driver is a compile error rather than an X in simulation.

- Explicit enum encodings (as in fecim_pkg.sv), so the STATUS register and HEX display show documented values instead of synthesizer-chosen ones.

- Every case on an enum gets a default. Unassigned enum values propagate X in simulation and the failure appears far from its cause.

2. Simulation toolchain

The language choice constrains this, so decide it now.

2.1 Verilator for Lane B

Icarus Verilog has weak SystemVerilog support and should not be used. Verilator has good coverage of the synthesizable subset and — the reason it is the right pick here — it compiles RTL into C++.

That means the golden reference model and the RTL can be linked into a single C++ test binary, stepping both and comparing accumulators cycle by cycle rather than only comparing final results over a serial link. The C++ reference model was already on the plan for the sim backend and the demo fallback; Verilator makes it the verification harness too, which is the fourth job that model is now doing.

tb/

├── cpp/

│   ├── model.cpp          # golden reference (Lane B)

│   ├── tb_mac_array.cpp   # steps Verilated RTL + model together

│   └── golden_packets.h   # generated fixtures, shared with Python

└── sv/

└── tb_uart.sv         # directed waveform tests (Questa)

2.2 Questa Intel Starter Edition for waveforms

Bundled with Quartus, good SystemVerilog simulation support, and the right tool when you need to look at a waveform — UART bit timing especially.

One limitation worth planning around: the Starter Edition does not include full constrained-random generation or functional coverage. Those are Questa Prime features. Lane B's verification plan should therefore rest on directed tests plus randomized stimulus driven from C++ under Verilator, not on SystemVerilog constrained-random. The test lists in the component specs are already written this way.

3. The package is the coordination mechanism

rtl/fecim_pkg.sv holds every parameter, type, and protocol encoding that crosses a module or work-lane boundary. Import it everywhere:

import fecim_pkg::*;

For a three-person team this is the highest-leverage file in the repository. The classic failure on a project like this is Lane A changing CMD_READ_RESULT from 0x05 to 0x06, Lane C not noticing, and two days disappearing into a bug that looks like a datapath problem. One declaration site makes that impossible on the RTL side.

3.1 The remaining gap, and how to close it

The package fixes RTL-to-RTL drift. It does not fix RTL-to-Python drift — Lane C's driver still encodes 0x05 independently.

Close it by making docs/protocol.yaml the source and generating both:

docs/protocol.yaml

├─(gen)─► rtl/fecim_pkg_gen.svh      imported by fecim_pkg.sv

└─(gen)─► sw/fecim/protocol.py       imported by the driver

The generator is about forty lines of Python and runs in CI, failing the build if the generated files are stale. It eliminates an entire category of integration bug, and it is a clean, self-contained task for whoever finishes their week-1 work first.

It also happens to be exactly the kind of artifact M0 wants in the repository — a planning document that turned into tooling, with a commit trail showing why.

4. Corrected snippets

These replace the Verilog fragments in the earlier component specs.

4.1 baud_gen (UART spec §3.2)

module baud_gen

import fecim_pkg::*;

(

input  logic                  clk,

input  logic                  rst_n,

input  logic [BAUD_ACC_W-1:0] baud_inc,

output logic                  tick_16x

);

logic [BAUD_ACC_W:0] acc;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) acc <= '0;

else        acc <= {1'b0, acc[BAUD_ACC_W-1:0]} + baud_inc;

end

assign tick_16x = acc[BAUD_ACC_W];

endmodule : baud_gen

4.2 RX synchronizer (UART spec §4.2)

logic [2:0] rx_sync;

logic       rx_bit;

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) rx_sync <= '1;                     // idle HIGH

else        rx_sync <= {rx_sync[1:0], rxd};

end

assign rx_bit = rx_sync[2];

Reset to '1, not '0. The line idles high, so a synchronizer that resets low presents a falling edge to the state machine on release — indistinguishable from a start bit, producing one garbage byte after every reset. This is a two-character fix and a genuinely confusing bug to chase, because it only appears on the first byte and looks like a host-side framing problem.

4.3 CRC-8 (parser spec §6)

Moved into fecim_pkg.sv as crc8_byte() so the parser and the response generator share one definition. function automatic matters — a static function with internal state is not reentrant and will produce wrong results if called from two places in the same timestep.

4.4 Lane pipeline depth (control FSM spec §4.2)

Now fecim_pkg::LANE_PIPE_DEPTH. The control FSM's hold count in ACC references the package parameter directly, so adding a pipeline stage to mac_lane updates the sequencer automatically. This is exactly the coupling the earlier spec flagged as silently-drops-the-last-rows if it drifts.

5. FSM template

Every state machine in the design follows this shape:

ctrl_state_e state, next_state;

// state register

always_ff @(posedge clk or negedge rst_n) begin

if (!rst_n) state <= CTRL_IDLE;

else        state <= next_state;

end

// next-state logic

always_comb begin

next_state = state;                    // default: hold

unique case (state)

CTRL_IDLE  : if (start_compute) next_state = CTRL_CLEAR;

CTRL_CLEAR :                    next_state = CTRL_ACC;

CTRL_ACC   : if (acc_done)      next_state = CTRL_DRAIN;

CTRL_DRAIN : if (drain_done)    next_state = (last_pass) ? CTRL_DONE

: CTRL_CLEAR;

CTRL_DONE  :                    next_state = CTRL_IDLE;

default    :                    next_state = CTRL_IDLE;

endcase

end

Two-block style with a defaulted next_state — the default assignment means a forgotten branch holds state rather than inferring a latch or falling to X. unique case adds a simulation assertion that the states are mutually exclusive and gives Quartus a one-hot hint.

6. Lane array instantiation

The one place generate earns its keep:

genvar l;

generate

for (l = 0; l < NUM_LANES; l++) begin : g_lane

mac_lane u_lane (

.clk        (clk),

.rst_n      (rst_n),

.w_addr     (seq_cnt),

.act        (act_bcast),

.noise_en   (noise_en),

.acc_clear  (acc_clear),

.acc_en     (acc_en),

.shift_en   (shift_en),

.shift_in   (lane_acc[l+1]),        // drain shift chain

.acc_out    (lane_acc[l])

);

end

endgenerate

assign lane_acc[NUM_LANES] = '0;                // chain tail

Named generate blocks (g_lane) matter — they produce readable hierarchical paths in Questa and in Quartus timing reports. Without the label you get genblk1[7] and lose an hour cross-referencing.

Note the shift chain wiring: lane l takes its shifted input from lane l+1, with the tail tied to zero, so results emerge from lane 0. Getting this backwards reverses column order in the result buffer — see control FSM spec, test 3.

7. Lint

Run Verilator in lint mode on every commit, in CI:

verilator --lint-only -Wall -Wno-fatal \

--top-module fecim_top \

rtl/fecim_pkg.sv rtl/**/*.sv

-Wall catches width mismatches, unused signals, and incomplete sensitivity — the width mismatch warnings in particular are worth heeding. A 24-bit accumulator silently truncated to 16 bits somewhere is a bug that produces plausible small numbers and survives casual testing all the way to the noise sweeps in week 9.