FeCIM Component Spec: Result Buffer and Readout

Module: result_buffer, argmax_unit, readout_ser Owner: Lane A (RTL), response format jointly with Lane C Depends on: mac_array (drain chain), control_fsm, packet_tx Milestone: Week 4 (buffer), Week 6 (argmax) Status: draft

1. Purpose and scope

Captures column results as they emerge from the drain shift chain, holds them until the next COMPUTE, and serializes them into response packets.

Also computes on-chip argmax. That started as a bandwidth optimization — the activation buffer analysis showed result readout is 68% of sweep traffic — but it turns into the more interesting measurement, for reasons in §5.

2. Storage

TILE_COLS × 32 bits. For the MVP that is 64 × 32 = 2,048 bits; one M9K in 256×32 mode holds 256 results, leaving room for a 256-column tile.

Simple dual-port: write port for the drain, read port for readout. The two phases never overlap, so this is a convenience rather than a requirement, but it costs nothing and removes the need to think about it.

ACC_W is 32 per the MAC array spec §2 correction, which is exactly the M9K's native word width in this mode. Convenient, not a coincidence — 24-bit would have wasted the block or needed packing.

3. Write during DRAIN

Lane l holds the accumulator for column c = p·L + l. The chain output is tapped from lane 0.

| drain_cnt | Chain output holds | Write address |
| --- | --- | --- |
| 0 | lane 0 (no shift yet) | p·L + 0 |
| 1 | lane 1, shifted in | p·L + 1 |
| … | … | … |
| L−1 | lane L−1 | p·L + L−1 |

So result_addr = {pass_idx, drain_cnt} — concatenation, no arithmetic, because L is a power of two. shift_en asserts from cycle 1 onward; lane 0's own value is captured on cycle 0 before any shifting.

Values pass through the shared ADC quantizer (MAC array spec §8) before being written, so the buffer holds post-ADC values.

4. Result validity

Two small counters, both worth their area:

results_valid — cleared by CTRL[1] and at the start of each COMPUTE, set on CTRL_DONE. Exposed in STATUS. A READ_RESULT against an invalid buffer still returns data, but the host can tell it is stale.

result_seq — an 8-bit counter incremented on every COMPUTE completion, exposed in STATUS and echoed in every readout response.

The sequence number earns its ten LEs the first time a COMPUTE response is lost and the driver retries. Without it, the host cannot distinguish "the compute happened and the response was dropped" from "the compute never ran," and it will happily read results belonging to the previous vector. That failure produces a plausible accuracy number that is silently wrong for one image in however-many-thousand — invisible in testing, and exactly the kind of thing that makes a sweep irreproducible.

Results persist until the next COMPUTE, so a retried READ_RESULT is always safe.

5. Argmax, and why it should track second place too

5.1 The basic unit

Comparison happens as values are written during DRAIN — they already stream past one per cycle, so no extra pass is needed.

always_ff @(posedge clk) begin

if (compute_start) begin

max_val  <= 32'sh8000_0000;

max_idx  <= '0;

end else if (result_we && (result_addr < active_cols)) begin

if ($signed(adc_out) > max_val) begin

max_val <= $signed(adc_out);

max_idx <= result_addr;

end

end

end

The active_cols guard is required, not defensive. Control FSM open question 3 proposed rounding active_cols up to a multiple of L and discarding extras on the host. That works for READ_RESULT, but argmax runs on-chip and would happily select a garbage column. This gate resolves that question: round up in the sequencer, mask in the argmax.

Tie-breaking uses strict >, which keeps the lowest index on a tie. This matches numpy.argmax. The C++ model must use the same rule — with quantized ADC output, ties are not rare, and a mismatched tie rule produces sporadic disagreements that look like noise model bugs.

5.2 Track the runner-up as well

Add a second comparator holding the second-largest value. About 70 more LEs.

The reason is not bandwidth. Decision margin — the gap between the best and second-best column — degrades continuously as noise increases, while classification accuracy is a step function that only moves when the margin crosses zero.

An accuracy-versus-noise plot is flat, flat, flat, then falls off a cliff. A margin-versus-noise plot shows the mechanism eroding smoothly the whole time, and makes the accuracy cliff explicable rather than merely observed. For the fatigue-cycling result in week 9 — the headline plot of the project — that is the difference between "accuracy drops after 10⁵ cycles" and "margin decays continuously from cycle one and crosses the decision threshold at 10⁵."

The second is a much stronger claim, and it costs one comparator.

It also has diagnostic value during bring-up: if margins are large but accuracy is poor, your weight mapping is wrong. If margins are near zero everywhere, your activation scaling is wrong. Accuracy alone cannot distinguish those.

5.3 ADC before argmax

Quantization is applied before the comparison, per MAC array spec open question 2.

Physically the ADC precedes any digital post-processing, so this is the correct order. It also means coarse ADC settings can flip the winner when two columns are close, which is real behaviour and shows up in the margin plot as a quantization floor.

6. Readout serialization

Results are 32-bit; UART is 8-bit. The serializer presents a byte stream to packet_tx, which computes CRC over it as it goes.

Little-endian, two's complement, matching struct.unpack('<i') on the host.

| Signal | Dir | Description |
| --- | --- | --- |
| rd_go | in | start, with range latched |
| rd_start | in | first word index |
| rd_count | in | number of words |
| rd_data | out | byte stream |
| rd_valid | out | byte available |
| rd_next | in | consumer took the byte |

The FSM reads one 32-bit word, emits four bytes with handshake, then reads the next. At ~4,340 clock cycles per UART byte, the single-cycle M9K read latency is invisible; do not build a prefetch.

6.1 Response formats

READ_RESULT — payload [start:2][count:2], response is count × 4 bytes. MAX_PAYLOAD = 512 allows 128 words per packet, so a 128-column tile needs one packet and a 256-column tile needs two.

READ_ARGMAX — no payload, response is 10 bytes:

| Offset | Width | Field |
| --- | --- | --- |
| 0 | 2 | max_idx |
| 2 | 4 | max_val (signed) |
| 6 | 4 | second_val (signed) |

Ten bytes rather than one. The bandwidth difference against the 1-byte version is negligible — 102 bytes per image versus 93 — and the margin comes free with it.

6.2 Updated sweep cost

| Transaction | Bytes |
| --- | --- |
| WRITE_ACT | 72 |
| COMPUTE request + response | 12 |
| READ_ARGMAX request + response | 18 |
| Total | 102 |

Versus 292 for full readout. At 921600 baud a 400-point sweep over 1,000 images runs in about 7.4 minutes, against 2.8 hours for the naive path. This is the number that makes week 9 feasible.

Keep full READ_RESULT — the equivalence tests need raw accumulators, and any plot that is not a classification metric needs the actual values.

7. Register and command additions

| CMD | Name | Payload | Response |
| --- | --- | --- | --- |
| 0x08 | READ_ARGMAX | — | 10 bytes (§6.1) |

STATUS (0x0A) gains:

| Bits | Field |
| --- | --- |
| [3:0] | FSM state |
| [4] | busy |
| [5] | last error |
| [6] | results_valid |
| [15:8] | result_seq |

8. Resource estimate

| Element | LEs | M9K |
| --- | --- | --- |
| Result M9K + write address | 25 | 1 |
| Argmax comparator + index | 70 | 0 |
| Second-place comparator | 70 | 0 |
| Readout FSM + byte serializer | 90 | 0 |
| result_seq, results_valid | 15 | 0 |
| Total | ~270 | 1 |

Design running total: ~10,800 LEs (22%), 35 M9K (19%), 72 multipliers (25%).

9. Verification

- Drain-to-buffer mapping. Distinct known value per column; confirm each lands at the correct buffer address. An off-by-one in drain_cnt shifts every column by one; a reversed shift chain mirrors them. Both are silent in aggregate metrics.

- Multi-pass addressing. Confirm pass 1 writes to addresses 32–63 and does not overwrite pass 0. Use values that make ordering errors obvious.

- Argmax against NumPy. 10,000 random result vectors, RTL against numpy.argmax, including deliberately constructed ties. Exact match on both index and value.

- Argmax masking. Set active_cols to a non-multiple of 32 (e.g. 48), leave garbage in columns 48–63, confirm argmax ignores them. Directed test for §5.1.

- Second-place correctness. Confirm second_val against a sorted reference, including the case where the top two values are equal.

- Byte order. Read a known result through the full serializer and unpack with struct.unpack('<i') in Python. Include negative values — a sign error only shows on those.

- Persistence and retry. Compute, read results, read again without recomputing, require identical output. Then confirm result_seq did not change.

- Stale detection. Compute, discard the response, compute again, and confirm result_seq advanced by two so the host can detect the gap.

- Range checking. READ_RESULT with start + count > TILE_COLS must return ST_ADDR_RANGE, not wrapped data.

10. Open questions for the freeze meeting

- Should READ_ARGMAX also return results_valid and result_seq? They are in STATUS, but including them costs 3 bytes and saves a round trip per image during sweeps — roughly 3% of sweep time. Marginal; decide on driver ergonomics.

- Top-k rather than top-2? Top-5 accuracy is a standard metric and would need a small sorting network. Probably not worth it for a 10-class digit task, but if the demo grows to a larger label set it becomes relevant.

- Should the buffer hold results from the previous compute as well? Two banks would let the host compare consecutive runs on-chip — e.g. the same input with noise on and off, returning only the difference. One extra M9K, and it would cut noise-sweep traffic further. Interesting, but the current numbers do not justify it.

- Does CTRL[1] need to physically zero the buffer, or is results_valid sufficient? Zeroing costs a 64-cycle sweep state in the FSM. The flag is probably enough, but zeroing makes debugging less confusing when stale data appears in a waveform.