FeCIM Component Spec: UART Transceiver

Module: uart_tx, uart_rx, baud_gen Owner: Lane A (RTL) Milestone: Week 2 (simulation), Week 3 (hardware gate) Status: draft — freeze before week 2

1. Purpose and scope

Byte-level serial transport between the host PC and the FPGA. This module knows nothing about packets, commands, or CRC — it moves bytes in and bytes out. All framing above the byte level belongs to packet_parser.

Deliberately excluded: FIFOs, flow control (RTS/CTS), parity, interrupt logic, multi-byte buffering.

2. Physical layer

2.1 Wiring

The DE10-Lite mini-USB port is USB-Blaster (JTAG) only — there is no USB-UART bridge on the board. A CP2102 or CH340 breakout connects to the Arduino Uno R3 expansion header.

| Dongle pin | DE10-Lite | Signal | Notes |
| --- | --- | --- | --- |
| TXD | ARDUINO_IO[0] | rxd (FPGA input) | crossover — dongle TX to FPGA RX |
| RXD | ARDUINO_IO[1] | txd (FPGA output) | crossover |
| GND | GND | — | required, common reference |
| VCC | do not connect | — | power the dongle from USB |

2.2 Voltage — read before wiring

The Arduino header I/O bank on DE10-Lite is 3.3 V. Many CP2102 and CH340 breakouts ship configured for 5 V logic or have a solder jumper selecting it. Applying 5 V to a MAX 10 I/O pin can damage it permanently.

Before connecting anything: set the dongle jumper to 3.3 V, then confirm with a meter that its idle TX line sits at ~3.3 V, not ~5 V. Leave the dongle's VCC pin disconnected — it draws power from USB, and tying two supplies together buys nothing but risk.

2.3 Pin assignments

# Verify against the DE10-Lite User Manual pin table before building —

# do not trust these from memory.

set_location_assignment PIN_AB5 -to rxd    ;# ARDUINO_IO[0]

set_location_assignment PIN_AB6 -to txd    ;# ARDUINO_IO[1]

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to rxd

set_instance_assignment -name IO_STANDARD "3.3-V LVTTL" -to txd

set_instance_assignment -name WEAK_PULL_UP_RESISTOR ON -to rxd

The weak pull-up on rxd matters: with the dongle unplugged the line floats, and a floating input will produce spurious start-bit detections. Pulled high, a disconnected line reads as idle.

2.4 Frame format

8N1 — one start bit (low), eight data bits LSB first, one stop bit (high), no parity. Idle state is high. Ten bit-times per byte.

3. Baud generation

3.1 Why not a plain integer divider

The obvious approach — count f_clk / (16 × baud) cycles per oversample tick — works acceptably at 115200 and badly everywhere else:

| Target baud | Integer divisor | Actual baud | Error |
| --- | --- | --- | --- |
| 115,200 | 27 | 115,741 | +0.47% |
| 230,400 | 14 | 223,214 | −3.1% |
| 460,800 | 7 | 446,429 | −3.1% |
| 921,600 | 3 | 1,041,667 | +13% |

Anything past 0.47% error starts eating the sampling margin, and 921600 is unusable. Since you'll want to raise the baud rate in week 8 to cut the 360 ms weight-load time, design for it now.

3.2 Fractional accumulator

A 16-bit phase accumulator, incremented every clock, emitting a tick on overflow:

reg [16:0] acc;                       // 17 bits: 16 phase + carry out

wire       tick_16x = acc[16];

always @(posedge clk or negedge rst_n)

if (!rst_n) acc <= 17'd0;

else        acc <= {1'b0, acc[15:0]} + baud_inc;

The increment is baud_inc = round(2^16 × 16 × baud / f_clk):

| Target baud | baud_inc | Actual baud | Error |
| --- | --- | --- | --- |
| 115,200 | 2416 | 115,204 | +0.003% |
| 230,400 | 4832 | 230,408 | +0.003% |
| 460,800 | 9664 | 460,816 | +0.003% |
| 921,600 | 19327 | 921,583 | −0.002% |

Cost is one 16-bit adder and register, roughly 35 LEs, in exchange for essentially exact baud at any rate. baud_inc becomes a config register, so the host can renegotiate speed at runtime without a rebuild.

Jitter. The fractional divider means individual tick intervals vary by up to one clock (20 ns) while the long-run average is exact. Since ticks are only counted to sixteen and the sample lands mid-bit, the jitter never accumulates. Harmless.

3.3 Shared generator, independent phase

One free-running baud_gen feeds both TX and RX. RX keeps its own modulo-16 phase counter that it resets on start-bit detection, so its sampling phase aligns to the incoming byte rather than to the free-running tick. Without that reset you inherit up to one tick (6.25% of a bit) of arbitrary phase error on every byte.

4. uart_rx

4.1 Ports

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | in | 1 | 50 MHz |
| rst_n | in | 1 | async assert, sync deassert |
| tick_16x | in | 1 | from baud_gen |
| rxd | in | 1 | asynchronous — from pin |
| rx_data | out | 8 | valid when rx_valid |
| rx_valid | out | 1 | single-cycle strobe |
| rx_frame_err | out | 1 | single-cycle, stop bit was low |

4.2 Input synchronization

rxd crosses from an unrelated clock domain (the dongle's) and must be synchronized before use. Three flops, not two — the extra stage is nearly free and the first stage of a 2-FF synchronizer on a slow asynchronous input is the classic source of intermittent byte corruption that shows up as a 1-in-10,000 failure rate two weeks before demo day.

reg [2:0] rx_sync;

always @(posedge clk) rx_sync <= {rx_sync[1:0], rxd};

wire rx_bit = rx_sync[2];

Mark this path with a false-path or set_max_delay constraint in the SDC so the fitter does not try to time it.

4.3 Sampling with majority voting

Each bit spans sixteen ticks. Rather than sampling once at tick 8, sample at ticks 7, 8, and 9 and take the majority. This is what 16550-family UARTs do, and it rejects single-tick glitches from cable noise for the cost of about 10 LEs.

tick:  0  1  2  3  4  5  6  7  8  9 10 11 12 13 14 15

^  ^  ^

└──┴──┴── majority vote

4.4 State machine

IDLE ──(rx_bit falls)──► START ──(vote@mid = 0)──► DATA

▲                        │                        │

│                        │ (vote@mid = 1)         │ (8 bits)

│                        │  false start           ▼

│◄───────────────────────┘                      STOP

│                                                 │

└──── rx_valid / rx_frame_err ◄───────────────────┘

- IDLE — wait for a high-to-low transition. Reset phase counter on detection.

- START — sample at mid-bit. If the line has returned high it was a glitch, not a start bit; return to IDLE without error. This rejects noise on an idle line.

- DATA — eight bits, LSB first, shifted into rx_data from the MSB end.

- STOP — sample mid-bit. High means a clean frame, assert rx_valid. Low means a framing error: assert rx_frame_err, and still return to IDLE rather than hunting for the next edge. Deliberately simple — the packet layer's CRC catches corruption, and the parser resynchronizes on the sync byte.

4.5 Error budget

A byte takes ten bit-times. The last bit is sampled 9.5 bit-times after the start edge, so sampling stays inside the correct bit as long as cumulative timing error stays under half a bit — about 5% per bit. With 0.003% baud error and 6.25% worst-case initial phase error from tick quantization, total worst case is roughly 6.3%, comfortably inside the window. The 3% baud error of the naive integer divider at 230400 would have consumed most of that margin.

5. uart_tx

5.1 Ports

| Signal | Dir | Width | Description |
| --- | --- | --- | --- |
| clk | in | 1 | 50 MHz |
| rst_n | in | 1 |  |
| tick_16x | in | 1 | from baud_gen |
| tx_data | in | 8 | captured on tx_start |
| tx_start | in | 1 | single-cycle, ignored while busy |
| tx_busy | out | 1 | high from load to stop-bit completion |
| txd | out | 1 | to pin, idle high |

5.2 Operation

On tx_start with tx_busy low, load a 10-bit shift register with {1'b1, tx_data[7:0], 1'b0} — stop bit, data, start bit — and shift out LSB first, one bit every sixteen ticks. txd is the shift register LSB. Assert tx_busy throughout.

txd must reset to 1. A reset that drives it low looks like a start bit to the host and will produce a spurious byte on every reset.

5.3 No FIFO

At 115200 baud a byte occupies 86.8 µs — 4,340 clock cycles. Every consumer and producer in this design operates in single-digit cycles, so a simple tx_busy handshake is sufficient and a FIFO would add area and a second failure mode for no benefit. The readout FSM waits on tx_busy between bytes.

Revisit only if you later add a streaming mode that must sustain back-to-back bytes with zero inter-byte gap. You will not need this.

6. Resource and timing

| Block | LEs (est.) | M9K | Mult |
| --- | --- | --- | --- |
| baud_gen | 35 | 0 | 0 |
| uart_rx | 60 | 0 | 0 |
| uart_tx | 45 | 0 | 0 |
| Total | ~140 | 0 | 0 |

About 0.3% of the 10M50. Longest combinational path is the 16-bit accumulator carry chain — trivially inside a 20 ns period. Expect first-pass timing closure; if this module fails timing, something is wrong elsewhere.

7. Verification

Lane B owns items 1–5; Lane A owns 6–7.

- Bit-banged RX. Testbench drives rxd at exactly 115200 with a known byte; check rx_data and single-cycle rx_valid. Sweep all 256 values.

- TX waveform. Capture txd, measure bit periods, confirm 8N1 framing and LSB-first ordering.

- Loopback. Wire txd to rxd in simulation, stream 1,000 random bytes, assert exact recovery.

- Baud tolerance. Drive rxd at ±2%, ±3%, ±4% of nominal. Should pass through ±3% at minimum. This test is what proves the error budget in §4.5 is real rather than arithmetic on paper.

- Fault injection. (a) Single-tick glitch mid-bit — majority vote must reject it. (b) Short low pulse on an idle line — must be rejected as a false start, no rx_valid. (c) Stop bit driven low — rx_frame_err asserts, rx_valid does not.

- Hardware loopback. Jumper ARDUINO_IO[0] to ARDUINO_IO[1], no dongle. Python is not involved. Confirms pin assignment and I/O standard independently of the host, which is worth doing before the dongle arrives.

- Hardware round trip. Real dongle, 10,000 random bytes echoed through Python, zero errors. Repeat at every baud rate you intend to support.

8. Week 3 gate

The project's first hard checkpoint. All of the following must hold:

- [ ] Dongle wired, verified at 3.3 V logic levels

- [ ] Hardware loopback (test 6) passes with a jumper

- [ ] Python pyserial sends a byte; the board echoes it

- [ ] 10,000-byte round trip at 115200 with zero errors

- [ ] HEX display shows RX byte count, LEDR shows activity

- [ ] Framing errors counted and visible

If this has not passed by end of week 4, escalate to the whole team. Everything downstream — parser, config registers, weight loading, every sweep — depends on this link working. The fallback is JTAG-to-Avalon via System Console, which costs you the clean Python driver and a week of TCL, so it is worth several days of debugging to avoid.

9. Open questions for the freeze meeting

- Is baud_inc a config register (runtime-settable) or a synthesis parameter? Runtime is more flexible but creates a chicken-and-egg problem — the host must already be talking at the old rate to change it. Suggest: parameter for the default, register for overrides, with a fixed 115200 fallback on reset.

- Should rx_frame_err increment a counter readable over IDENTIFY? Cheap, and very useful when debugging cable and baud problems in week 3.

- Do we want a break-detect (line low longer than one frame) as a hardware resynchronize trigger? Probably unnecessary given CRC and sync-byte recovery at the packet layer.